elf version 2.3

-kb      Enable ascii keyboard

-a       Disable address display in run mode (improves performance)

-c freq  Set frequency of clock

-d       Enable debugging mode

-e       Enable extended ports

-lm      Enable LED Module

-m       Use extended ram pager (1mb), requires extended ports

-ref4    Reverse sense of EF4

-1861    Enable 1861 Pixie graphics

-6847    Enable 6847 graphics

-6847=32 Enable 6847 graphics in 32 column mode

-k       Enable hex keybad (like elf II)

-p       Enable printer output to printer.out

-r name  read specified program into memory

-R name  Read specified program as rom

-t name  use file 'name' to hit keys during run mode

-v       Show program version

-vt100   Enable vt100 terminal

-nk      Set memory size to nk (example: -32k)

-elf2    Simulate an Elf II

-super   Simulate a Super Elf

-studio2 Simulate a Studio II, this also sets -1861, -c 1.789, -a -4k

-st2 name Read program file in .st2 format

-1805    Enable 1805 instructions (timer not currently supported)

-uc      Force uppercase when reading keyboard

-fdc     Enable disk 1793 controller

-ide     Enable IDE disk controller

This program is a simulation of an early computer.  The computer in this

simulation is similar in design to the Elf computer project published in

popular electronics in 1976.  The base computer has 256 bytes of ram, 8

toggle switches for input, 4 hex displays to show the current address and

2 hex displays for data output.

Front panel swithes (Standard Elf):

IN   - This button initiates a DMA IN operation of the processor.  The

values of the 8 data switches will be written into the memory

pointed to by R[0].

LOAD - This switch when up, and RUN down places the computer into load

mode.  In this mode, values can be set on the 8 data switches and

then loaded into memory by pressing the IN button.  After the

transfer, the address displays will show the address just written

and the data displays will show the byte written.

MP   - This switch when up disables memory writes.  This would allow you

to step through memory in the LOAD mode without actually modifying

memory with each IN press, but would still increment the address.

using this you can step to an address in memory, turn MP off and

then write a new value into memory.

RUN  - This swithch when up and LOAD down places the cpu into run mode

the program will begin to execute from address 0.  When both RUN

and LOAD are down, the cpu is put into the reset state, setting the

current address back to 0, and disabling DMA and INT operations.

ON   - Turning this switch off will end the program.

DATA - These 8 switches are used to specify a byte to be input to memory.

During RUN mode, reading port 4 will also read the value of these

switches.

EF   - These 4 switches are connected to the 4 ef inputs of the cpu.

Front panel switches (Super Elf):

L    - Places the computer into Load mode

R    - Places the cpu into the reset state

G    - Go, this starts the processor running

W    - Enter Wait mode.  This also removes memory protection in Load mode

M    - Enable 32byte mini monitor, this is in address 00-1f

S    - Single step the processor, will execute next instruction and then

enter Wait mode

P    - Enables memory Protection in Load mode

I    - Input, this is connected to DMA_IN in Load mode, or EF4 in run mode

Note on the Vt-100 emulation:

Currently the vt-100 emulation is very primative.  Most escape sequences have

not yet been implemented. the following escape sequences are currently

implemented:

^[nA

^[nB

^[nC

^[nD

^[n;nH

^[n;nf

^[J

^[1J

^[2J

^[K

^[1K

^[2K

^[s

^[u

The vt-100 emulation is based upon the serial model of the Elf II Giant Board.

Serial output is on the Q line, serial input is on the EF4 line.  No uart exists,

the timing must be generated in software.  See the source for IDIOT/4 to get an

understanding of how this software uart works.

Debugger:

---------

The debugger screen gives you an inside view of what is going on in the Elf

emulator.  The F1 key enables/disables the debugger screen.  If a -d is

included on the command line then the debugger window will be opened on

startup.  The following commands are available in the debugger:

?                       - Show help

AS addr                 - Assemble

DA [addr][,cnt]         - DisAssemble

DB                      - Display breakpoints

DM [addr][,cnt]         - Display Memory

DP                      - Display current memory Pointer

RB addr                 - Remove Breakpoint

RN [addr]               - Run.  addr as optional start address

SB addr                 - Set breakpoint

SM addr byte byte ...   - Set Memory to byte list

SR reg value            - Set Register (P,X,D,DF,T,Q,R0-RF)

ST [num]                - Step 1 or num instructions (num=dec)

TR ON | OFF             - Turn tracing on or off

TRAP asm_cmd            - Trap on specified command

RT asm_cmd              - Remove specified command trap.

TREG reg value          - Set a register trap

RR reg value            - Remove a register trap

The value of 'addr' can be:

P      - The value of R(P)

X      - The value of R(X)

Rn     - The value of R(n), example R1, RA, etc

hexnum - up to 4 digit hexadecimal number

Devices:

--------

Pixie Graphics (-1861)

input port   1         Enable Graphics

output port  1         Disable Graphics

flag line    EF1       In Frame Indicator

Keyboard (-k)

input port   7         Read Data

flag line    EF3       Data Ready

PortExtender (-e)

output port  5         Port selector

output port  6         Write selected port

input port   6         Read selected port

Printer (-p)

output port  7         Write Data

Led Module (-lm)

output port  4         Write Data

Vt100 (-vt100)

serial out   Q         Serial Out

flag line    EF4       Serial In

FDC (-fdc)

output port  2         Select fdc port

output port  3         Write selected port

input  port  3         Read selected port

flag line    2         FDC Data request line

IDE (-ide)

output port  2         Select ide port

output port  3         Write selected port

input  port  3         Read selected port

The simulator now allows you to reassign device port assignments when the

program is executed.  The following flags can now be used when specifying

device options:

In    - Redesignate an Input port to port 'n'

On    - Redesignate an Output port to port 'n'

En    - Redesignate an EF line to line 'n'

These options are place right after the device switch. here are some examples:

-p=o2         - Enables printer on output port 2

-kb=i2,e2     - Enables keyboard on input port 2 and flag line 2

-e=o1,o7      - Enable extended ports, port select on port 1,write selected

is port 7, read selected port would remain on 6

Special Registers:

------------------

Register       Purpose

R(0)           DMA in or out

R(1)           Interrupt vector

R(2)           X register during interrupt

Registers:

D   8-bits     Data Register

DF  1-bit      Data Flag (Carry)

R   16-bits    16 Scratchpad Registers

P   4-bits     Specifies which R register is program counter

X   4-bits     Specifies which R register is data pointer

N   4-bits     Low order nybble of instruction

T   8-bits     Holds X and P during interrupt. X is high nybble

IE  1-bit      Interrupt Enable

Q   1-bit      Output flip-flop

Instruction Set:

Opcode  Mnemonic  Instruction             Operation

===========================================================================

0N       LDN  Load via N              M(R(N))->D; For N not 0

4N       LDA  Load Advance            M(R(N))->D; R(N)+1->R(N)

F0       LDX  Load via X              M(R(X))->D

72      LDXA  Load via X and advance  M(R(X))->D; R(X)+1->R(X)

F8       LDI  Load immediate          M(R(P))->D; R(P)+1->R(P)

5N       STR  Store via N             D->M(R(N))

73      STXD  Store Via X and dec.    D->M(R(X)); R(X)-1->R(X)

1N       INC  Increment reg N         R(N)+1->R(N)

2N       DEC  Decrement reg N         R(N)-1->R(N)

60       IRX  Increment reg X         R(X)+1->R(X)

8N       GLO  Get low reg N           R(N).0->D

AN       PLO  Put low reg N           D->R(N).0

9N       GHI  Get high reg N          R(N).1->D

BN       PHI  Put high reg N          D->R(N).1

------------------------------------------------------------------------

F1        OR  Or                      M(R(X)) or D->D

F9       ORI  Or immediate            M(R(P)) or D->D; R(P)+1->R(P)

F3       XOR  Exclusive or            M(R(X)) xor D->D

FB       XRI  Exclusive or immediate  M(R(P)) xor D->D; R(P)+1->R(P)

F2       AND  And                     M(R(X)) and D->D

FA       ANI  And immediate           M(R(P)) and D->D; R(P)+1->R(P)

F6       SHR  Shift right             Shift D right; lsb(D)->DF;

0->msb(D)

76      SHRC  Shift right with carry  Shift D right; lsb(D)->DF;

DF->msb(D)

FE       SHL  Shift left              Shift D left; msb(D)->DF;

0->lsb(D)

7E      SHLC  Shift left with carry   Shift D left; msb(D)->DF;

DF->lsb(D)

F4       ADD  Add                     M(R(X))+D->DF,D

FC       ADI  Add immediate           M(R(P))+D->DF,D; R(P)+1->R(P)

74       ADC  Add with carry          M(R(X))+D+DF->DF,D

7C      ADCI  Add with carry imm.     M(R(P))+D+DF->DF,D; R(P)+1->R(P)

F5        SD  Subtract D              M(R(X))-D->DF,D

FD       SDI  Subtract D immediate    M(R(P))-D->DF,D; R(P)+1->R(P)

75       SDB  Sub. D with borrow      M(R(X))-D-DF->DF,D

7D      SDBI  Sub. D with borrow imm. M(R(P))-D-DF->DF,D; R(P)+1->R(P)

F7        SM  Subtract memory         D-M(R(X))->DF,D

FF       SMI  Subtract Mem. imm.      D-M(R(P))->DF,D; R(P)+1->R(P)

77       SMB  Sub. Mem. with borrow   D-M(R(X))-DF->DF,D;

7F      SMBI  Sub. Mem. w/borrow imm. D-M(R(P))-DF->DF,D; R(P)+1>R(P)

------------------------------------------------------------------------

30        BR  Branch                  M(R(P))->R(P).0

38       NBR  No Branch               R(P)+1->R(P)

32        BZ  Branch if D=0           If D=0, M(R(P))->R(P).0

else R(P)+1->R(P)

3A       BNZ  Branch if D<>0          If D<>0, M(R(P))->R(P).0

else R(P)+1->R(P)

33       BDF  Branch if DF=1          if DF=1, M(R(P))->R(P).0

else R(P)+1->R(P)

3B       BNF  Branch if DF=0          if DF=0, M(R(P))->R(P).0

else R(P)+1->R(P)

31        BQ  Branch if Q=1           if Q=1, M(R(P))->R(P).0

else R(P)+1->R(P)

39       BNQ  Branch if Q=0           if Q=0, M(R(P))->R(P).0

else R(P)+1->R(P)

34        B1  Branch if EF1=1         if EF1=1, M(R(P))->R(P).0

else R(P)+1->R(P)

3C       BN1  Branch if EF1=0         if EF1=0, M(R(P))->R(P).0

else R(P)+1->R(P)

35        B2  Branch if EF2=1         if EF2=1, M(R(P))->R(P).0

else R(P)+1->R(P)

3D       BN2  Branch if EF2=0         if EF2=0, M(R(P))->R(P).0

else R(P)+1->R(P)

36        B3  Branch if EF3=1         if EF3=1, M(R(P))->R(P).0

else R(P)+1->R(P)

3E       BN3  Branch if EF3=0         if EF3=0, M(R(P))->R(P).0

else R(P)+1->R(P)

37        B4  Branch if EF4=1         if EF4=1, M(R(P))->R(P).0

else R(P)+1->R(P)

3F       BN4  Branch if EF4=0         if EF4=0, M(R(P))->R(P).0

else R(P)+1->R(P)

------------------------------------------------------------------------

C0       LBR  Long Branch             M(R(P))->R(P).1;

M(R(P)+1)->R(P).0

C8      NLBR  No long branch          R(P)+2->R(P)

C2       LBZ  Branch if D=0           if D=0 then M(R(P))->R(P).1;

M(R(P)+1)->R(P).0

else R(P)+2->R(P)

CA      LBNZ  Branch if D<>0          if D<>0 then M(R(P))->R(P).1;

M(R(P)+1)->R(P).0

else R(P)+2->R(P)

C3      LBDF  Branch if DF=1          if DF=1 then M(R(P))->R(P).1;

M(R(P)+1)->R(P).0

else R(P)+2->R(P)

CB      LBNF  Branch if DF=0          if DF=0 then M(R(P))->R(P).1;

M(R(P)+1)->R(P).0

else R(P)+2->R(P)

C1       LBQ  Branch if Q=1           if Q=1 then M(R(P))->R(P).1;

M(R(P)+1)->R(P).0

else R(P)+2->R(P)

C9      LBNQ  Branch if Q=0           if Q=0 then M(R(P))->R(P).1;

M(R(P)+1)->R(P).0

else R(P)+2->R(P)

------------------------------------------------------------------------

CE       LSZ  Skip if D=0             if D=0, R(P)+2->R(P)

else Continue

C6      LSNZ  Skip if D<>0            if D<>0, R(P)+2->R(P)

else Continue

CF      LSDF  Skip if DF=1            if DF=1, R(P)+2->R(P)

else Continue

C7      LSNF  Skip if DF=0            if DF=0, R(P)+2->R(P)

else Continue

CD       LSQ  Skip if Q=1             if Q=1, R(P)+2->R(P)

else Continue

C5      LSNQ  Skip if Q=0             if Q=0, R(P)+2->R(P)

else Continue

CC      LSIE  Skip if IE=1            if IE=0, R(P)+2->R(P)

else Continue

------------------------------------------------------------------------

00       IDL  Idle                    Wait for DMA or Interrupt

M(R(0))->Bus

C4       NOP  No operation            Continue

DN       SEP  Set P                   N->P

EN       SEX  Set X                   N->X

7B       SEQ  Set Q                   1->Q

7A       REQ  Reset Q                 0->Q

78       SAV  Save                    T->M(R(X))

79      MARK  Push X,P to stack       (X,P)->T; (X,P)->M(R(2)) then

P->X; R(2)-1->R(2)

70       RET  Return                  M(R(X))->(X,P); R(X)+1->R(X); 1->IE

71       DIS  Disable                 M(R(X))->(X,P); R(X)+1->R(X); 0->IE

------------------------------------------------------------------------

61       OUT  Output 1                M(R(X))->Bus; R(X)+1->R(X); Nlines=1

62       OUT  Output 2                M(R(X))->Bus; R(X)+1->R(X); Nlines=2

63       OUT  Output 3                M(R(X))->Bus; R(X)+1->R(X); Nlines=3

64       OUT  Output 4                M(R(X))->Bus; R(X)+1->R(X); Nlines=4

65       OUT  Output 5                M(R(X))->Bus; R(X)+1->R(X); Nlines=5

66       OUT  Output 6                M(R(X))->Bus; R(X)+1->R(X); Nlines=6

67       OUT  Output 7                M(R(X))->Bus; R(X)+1->R(X); Nlines=7

69       INP  Input 1                 Bus->M(R(X)); Bus->D; Nlines=1

6A       INP  Input 2                 Bus->M(R(X)); Bus->D; Nlines=2

6B       INP  Input 3                 Bus->M(R(X)); Bus->D; Nlines=3

6C       INP  Input 4                 Bus->M(R(X)); Bus->D; Nlines=4

6D       INP  Input 5                 Bus->M(R(X)); Bus->D; Nlines=5

6E       INP  Input 6                 Bus->M(R(X)); Bus->D; Nlines=6

6F       INP  Input 7                 Bus->M(R(X)); Bus->D; Nlines=7

------------------------------------------------------------------------

1805 Mode Instruction Set:

Opcode  Mnemonic  Instruction             Operation

===========================================================================

68 CN      RLDI  Register load immed.    M(R(P))->R(N).1; M(R(P)+1)->R(N).0

R(P)+2->R(P)

68 6N      RLXA  Register load via X     M(R(X))->R(N).1; M(R(X)+1)->R(N).0

and advance             R(X)+2->R(X)

68 AN      RSXD  Register store via X    R(N).0->M(R(X)); R(N).1->M(R(X)-1)

and decrement           R(X)-2->R(X)

68 2N      DBNZ  Decrement reg N and     R(N)-1->R(N) if R(N) not 0,

long branch if not 0    M(R(P))->R(P).1,M(R(P)+1)->R(P).0

else R(P)+2->R(P)

68 8N      RNX   Register N to X copy    R(N)->R(X)

------------------------------------------------------------------------

68 F4      DADD  Decimal Add             M(R(X))+D->DF,D, decimal adjust

68 FC      DADI  Decimal add immediate   M(R(P))+D->DF,D, decimal adjust

R(P)+1->R(P)

68 74      DADC  Decimal add w/carry     M(R(X))+D+DF->DF,D; decimal adjust

68 7C      DACI  Decimal add immediate   M(R(P))+D+DF->DF,D; decimal adjust

with carry              R(P)+1->R(P)

68 F7      DSM   Decimal sub memory      D-M(R(X))->DF,D; decimal adjust

68 FF      DSMI  Decimal sub memory      D-M(R(P))->DF,D; decimal adjust

immediate               R(P)+1->R(P)

68 77      DSMB  Desimal sub memory      D-M(R(X))-(not DF)->DF,D

with borrow             decimal adjust

68 7F      DSBI  Desimal sub immediate   D-M(R(P))-(not DF)->DF,D

with borrow             decimal adjust; R(P)+1->R(P)

------------------------------------------------------------------------

68 3E      BCI   Short branch on         if CI=1,M(R(P))->R(P).0; 0->CI

counter interrupt       else R(P)+1->R(P)

68 3F      BXI   Short branch on         if XI=1,M(R(P))->R(P).0; 0->CI

external interrupt      else R(P)+1->R(P)

------------------------------------------------------------------------

68 06      LDC   Load counter            Cntr stopped:D->CH,CNTR;0->CI

Cntr running:d->CH

68 08      GEC   Get counter             Cntr->D

68 00      STPC  Stop counter            Stop Cntr; 0->/32 prescaler

68 01      DTC   Decrement counter       Cntr-1->Cntr

68 07      STM   Set timer mode and      TPA/32->CNTR

start

68 05      SCM1  Set counter mode 1 and  /EF1->Cntr clock

start

68 03      SCM2  Set counter mode 2 and  /EF2->Cntr clock

start

68 04      SPM1  Set pulse width mode 1  TPA.EF1->Cntr clock

and start               /EF1 / stops count

68 02      SPM2  Set pulse width mode 2  TPA.EF2->Cntr clock

and start               /EF2 / stops count

68 09      ETQ   Enable toggle Q         if Cntr=01; next cntr clock /

/Q->Q

------------------------------------------------------------------------

68 0A      XIE   External int. enable    1->XIE

68 0B      XID   External int. disable   0->XIE

68 0C      CIE   External int. enable    1->CIE

68 0D      CID   External int. disable   0->CIE

68 76      DSAV  Save T,D,DF             R(X)-1->R(X); T->M(R(X))

R(X)-1->R(X); D->M(R(X))

Shift D right with carry

R(X)-1->R(X); D->M(R(X))

------------------------------------------------------------------------

68 8N      SCAL  Standard call           R(N).0->M(R(X)); R(N).1->M(R(X)-1);

R(X)-2->R(X); R(P)->R(N);

M(R(N))->R(P).1

M(R(N)+1)->R(P).0;

RN(N)+2->R(N)

68 9N      SRET  Standard return         R(N)->R(P)

M(R(X)+1)->R(N).1; M(R(X)+2)->R(N).0

R(X)+2->R(X)

------------------------------------------------------------------------

Ports as assigned by XElf

INP1    Activate pixie graphics (when loaded)

INP3    Read selected disk controller port

INP4    read data toggle switches

INP6    Read from selected extended port

INP7    read from ascii keyboard

OUT1    De-activate pixie graphics (when loaded)

OUT2    Write disk controller selection port

OUT3    Write to selected disk controller port

OUT4    write to data hex displays

OUT5    Write extended port select

OUT6    Write to selected extended port

OUT7    write to parallel port

Extended Ports:

1-15    Memory pager cells

16      DMA selection

DMA Selector

bits 0-2  select dma source

bit  7    select direction 0=read 1=write

DMA Channels

0       Disk Controller

Input Lines used by XElf

EF1     Pixie graphic frame indicator, 0=in frame, 1=out of frame

EF2

EF3     Ascii Keyboard data strobe (0=data ready)

EF4     Connected to IN button

Connected to serial-in when vt100 is enabled

Output Lines used by XElf

Q     Connected to serial-out when vt100 is enabled

Disk controller ports:

----------------------

Read      Write

0 - Status    command

1 - Track     track

2 - Sector    Sector

3 - Data      Data

4 -           Drive Select

Disk controller commands:

-------------------------        Bits

Type   Command          7  6  5  4  3  2  1  0

I    Restore          0  0  0  0  h  V r1 r0

I    Seek             0  0  0  1  h  V r1 r0

I    Step             0  0  1  u  h  V r1 r0

I    Step In          0  1  0  u  h  V r1 r0

I    Step Out         0  1  1  u  h  V r1 r0

II    Read Sector      1  0  0  m  0  E  0  0

II    Write Sector     1  0  1  m  0  E  0  a

III    Read Address     1  1  0  0  0  E  0  0

III    Read Track       1  1  1  0  0  E  0  0

III    Write Track      1  1  1  1  0  E  0  0

IV     Force Interrupt  1  1  0  1 i3 i2 i1 i0

h = 0  - Do not load head at beginning of command

h = 1  - Loas head at beginning of Command

V = 1  - Verify head position

V = 0  - Do not verify head position

u = 0  - Do not update track register

u = 1  - Update track register

m = 0  - Single sector read

m = 1  - Multi sector read

E = 0  - No 15ms delay

E = 1  - 15 ms delay

a = 0  - FB data mark

a = 1  - F8 Deleted data mark

r1 r0  = 0 0  - 6 ms step rate

= 0 1  - 12 ms step rate

= 1 0  - 20 ms step rate

= 1 1  - 30 ms step rate

Note: Currently only type I and II commands are supported.  Multi-sector

read/writes are also not yet supported.

Status:

-------

+---+-------------+-----------+-----------+-----------+-----------+------------+

|Bit| Type I      | Read Addr | Read Sec. | Write Sec | Read Track| Write trk  |

+---+-------------+-----------+-----------+-----------+-----------+------------+

| 0 | busy        | busy      | busy      | busy      | busy      | busy       |

| 1 | index       | drq       | drq       | drq       | drq       | drq        |

| 2 | track 0     | lost data | lost data | lost data | lost data | lost data  |

| 3 | CRC error   | CRC error | CRC error | CRC error | 0         | 0          |

| 4 | Seek error  | rnf       | rnf       | rnf       | 0         | 0          |

| 5 | Head loaded | 0         | rec type  | write flt | 0         | write fault|

| 6 | Write prot  | 0         | 0         | write prot| 0         | write prot |

| 7 | Not ready   | Not ready | Not ready | Not ready | Not ready | Not ready  |

+---+-------------+-----------+-----------+-----------+-----------+------------+